# OTP Voting Machine — Arduino Code

File: voting_machine.ino

Fixed Pin Mapping: Buzzer→A1, Green LED→A2, Red LED→A0

/*

=====================================================

OTP-Based Voting Machine — Arduino Uno

=====================================================

Fixed Pin Mapping (D0/D1 are Serial TX/RX — avoid!):

Buzzer   → A1  (was D1)

Green LED→ A2  (was D0)

Red LED  → A0  (unchanged)

=====================================================

*/

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <RTClib.h>

#include <Keypad.h>

#include <SPI.h>

#include <SD.h>

// ---- Pin Definitions (FIXED) ----

#define BUZZER_PIN  A1   // ← changed from D1 (Serial TX)

#define LED_GREEN   A2   // ← changed from D0 (Serial RX)

#define LED_RED     A0   // unchanged

#define SD_CS       10

// ---- LCD I2C (address 0x27 or 0x3F) ----

LiquidCrystal_I2C lcd(0x27, 16, 2);

// ---- RTC ----

RTC_DS3231 rtc;

// ---- Keypad (4x4) ----

const byte ROWS = 4, COLS = 4;

char keys[ROWS][COLS] = {

{'1','2','3','A'},

{'4','5','6','B'},

{'7','8','9','C'},

{'*','0','#','D'}

};

byte rowPins[ROWS] = {5, 6, 7, 8};   // Keypad Rows → D5–D8

byte colPins[COLS] = {2, 3, 4, 9};   // Keypad Cols → D2–D4, D9

Keypad keypad = Keypad(makeKeymap(keys), rowPins, colPins, ROWS, COLS);

// ---- TOTP Secret Key ----

// Simple 6-digit OTP derived from time + secret

const uint32_t SECRET = 123456UL; // Change this to any number

// ---- Voting State ----

int candidate = 0;

bool waitingForOTP = false;

String enteredOTP = "";

// =========================================

// Generate simple TOTP (30-second window)

// =========================================

String generateOTP() {

DateTime now = rtc.now();

uint32_t timeStep = now.unixtime() / 30;

uint32_t otp = (timeStep ^ SECRET) % 1000000;

String otpStr = String(otp);

while (otpStr.length() < 6) otpStr = "0" + otpStr;

return otpStr;

}

// =========================================

// Log vote to SD Card

// =========================================

void logVote(int cand, String otp) {

DateTime now = rtc.now();

File f = SD.open("votes.txt", FILE_WRITE);

if (f) {

f.print(now.timestamp());

f.print(" | Candidate: ");

f.print(cand);

f.print(" | OTP: ");

f.println(otp);

f.close();

}

}

// =========================================

// Feedback helpers

// =========================================

void buzzSuccess() {

digitalWrite(LED_GREEN, HIGH);

tone(BUZZER_PIN, 1000, 500);

delay(2000);

digitalWrite(LED_GREEN, LOW);

}

void buzzFail() {

digitalWrite(LED_RED, HIGH);

tone(BUZZER_PIN, 300, 800);

delay(2000);

digitalWrite(LED_RED, LOW);

}

void showWelcome() {

lcd.clear();

lcd.setCursor(0, 0); lcd.print("VOTING MACHINE");

lcd.setCursor(0, 1); lcd.print("Press 1-4 to vote");

}

// =========================================

// Setup

// =========================================

void setup() {

pinMode(BUZZER_PIN, OUTPUT);

pinMode(LED_GREEN, OUTPUT);

pinMode(LED_RED, OUTPUT);

lcd.init();

lcd.backlight();

if (!rtc.begin()) {

lcd.clear();

lcd.print("RTC Error!");

while (1);

}

if (!SD.begin(SD_CS)) {

lcd.clear();

lcd.print("SD Error!");

while (1);

}

showWelcome();

}

// =========================================

// Main Loop

// =========================================

void loop() {

char key = keypad.getKey();

if (!key) return;

if (!waitingForOTP) {

// ── Step 1: Voter selects candidate (1–4) ──

if (key >= '1' && key <= '4') {

candidate = key - '0';

String currentOTP = generateOTP();

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Candidate: " + String(candidate));

lcd.setCursor(0, 1); lcd.print("OTP: " + currentOTP);

delay(3000);  // show OTP for 3 seconds

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Enter OTP:");

lcd.setCursor(0, 1); lcd.print("");

enteredOTP = "";

waitingForOTP = true;

}

} else {

// ── Step 2: Voter enters 6-digit OTP ──

if (key == '*') {

// Clear entry

enteredOTP = "";

lcd.setCursor(0, 1); lcd.print("      ");

lcd.setCursor(0, 1);

} else if (key == '#') {

// Confirm and validate

String validOTP = generateOTP();

if (enteredOTP == validOTP) {

logVote(candidate, enteredOTP);

lcd.clear();

lcd.setCursor(0, 0); lcd.print("VOTE CAST!");

lcd.setCursor(0, 1); lcd.print("Thank you :)");

buzzSuccess();

} else {

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Invalid OTP!");

lcd.setCursor(0, 1); lcd.print("Try again");

buzzFail();

}

// Reset state

waitingForOTP = false;

candidate = 0;

enteredOTP = "";

showWelcome();

} else if (enteredOTP.length() < 6 && isDigit(key)) {

enteredOTP += key;

lcd.setCursor(enteredOTP.length() - 1, 1);

lcd.print('*');

}

}

}