# OTP Voting Machine — Connection Table

⚠  Important Fix:

D0 and D1 on Arduino Uno are Serial RX/TX pins. They CANNOT be used for LEDs/Buzzer.

These have been moved to A1 (Buzzer) and A2 (Green LED).

## Pin Connection Table

| Arduino Pin | Connects To | Wire Color | Notes |
| --- | --- | --- | --- |
| 5V | LCD VCC, RTC VCC | Red |  |
| 3.3V | SD Card VCC | Red | ⚠ NOT 5V — damages module! |
| GND | All GND pins | Black | Common ground |
| A4 (SDA) | LCD SDA + RTC SDA | Green | Shared I2C bus |
| A5 (SCL) | LCD SCL + RTC SCL | Yellow | Shared I2C bus |
| A1 ✅ | Buzzer + (via 100Ω) | Orange | Fixed — was D1 (Serial TX) |
| A2 ✅ | Green LED (via 220Ω) | Green | Fixed — was D0 (Serial RX) |
| A0 | Red LED + (via 220Ω) | Red |  |
| D2 | Keypad Col 1 | Purple |  |
| D3 | Keypad Col 2 | Purple |  |
| D4 | Keypad Col 3 | Purple |  |
| D5 | Keypad Row 1 | Orange |  |
| D6 | Keypad Row 2 | Orange |  |
| D7 | Keypad Row 3 | Orange |  |
| D8 | Keypad Row 4 | Orange |  |
| D9 | Keypad Col 4 | Purple |  |
| D10 | SD Card CS | Green | SPI Chip Select |
| D11 | SD Card MOSI | Green | SPI data out |
| D12 | SD Card MISO | Green | SPI data in |
| D13 | SD Card SCK | Green | SPI clock |

## Components Required

- Arduino Uno R3

- 16x2 LCD with I2C backpack

- RTC DS3231 module

- 4x4 Matrix Keypad

- SD Card module (SPI)

- Active Buzzer

- 1x Green LED + 220Ω resistor

- 1x Red LED + 220Ω resistor

- 100Ω resistor (buzzer)

- Jumper wires + Breadboard