# OTP-Based Voting Machine — Arduino Uno

## Short Description (paste on GitHub)

OTP-based electronic voting machine using Arduino Uno with 4x4 keypad, 16x2 LCD, RTC DS3231, and SD card logging.

## Full Description

This project is a secure electronic voting machine built using Arduino Uno. Voters select a candidate (1-4) using a 4x4 keypad. A 6-digit OTP (One-Time Password) is generated using the DS3231 real-time clock and displayed on a 16x2 I2C LCD for 3 seconds. The voter then enters the OTP to confirm their vote. A green LED and buzzer confirm a successful vote, while a red LED and buzzer reject a wrong OTP. All votes are saved with timestamps to an SD card.

Analog pins A0, A1, and A2 are used for the LEDs and buzzer to avoid conflicts with Arduino Serial pins D0 and D1.

## GitHub Tags to Add

arduino   voting-machine   otp   keypad   lcd   rtc   sd-card   electronics   embedded-systems   arduino-uno

## How It Works

- 1. Voter presses 1-4 to select a candidate

- 2. OTP is generated (6-digit, 30-second window using RTC)

- 3. OTP is shown on LCD for 3 seconds

- 4. Voter enters OTP using keypad (# to confirm, * to clear)

- 5. If OTP matches: Vote logged to SD card + green LED + beep

- 6. If wrong OTP: Rejected + red LED + long beep

- 7. Machine resets for next voter

## Libraries Required

- LiquidCrystal_I2C by Frank de Brabander

- RTClib by Adafruit

- Keypad by Mark Stanley

- SD (built-in with Arduino IDE)

- SPI (built-in with Arduino IDE)

- Wire (built-in with Arduino IDE)